library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity state is

port(clk, reset: in std_logic;

light: out std_logic_vector(11 downto 0);

ew_high, ns_high: out std_logic_vector(3 downto 0);

ew_low, ns_low: out std_logic_vector(3 downto 0));

end state;

architecture bhv of state is

type s is(s0, s1, s2, s3);

signal current_state: s;

signal east_red, east_yellow, east_green: std_logic; --东方三种灯

signal west_red, west_yellow, west_green: std_logic; --西方三种灯

signal north_red, north_yellow, north_green: std_logic; --北方三种灯

signal south_red, south_yellow, south_green: std_logic; --南方三种灯

signal tmp1,tmp2,tmp3,tmp4:std_logic_vector(3 downto 0);

begin

process(reset, clk, current_state)

begin

if(clk = '1' and clk'event) then --当clk 处于上升沿

if reset = '1' then

current_state <= s0; --复位时强行进入s0状态

tmp1<= "0000";

tmp2 <= "0101";

tmp3 <= "0000";

tmp4 <= "0101";

else

if(tmp1 = "0000" and not(tmp2 = "0000")) then --如果低位不够减，则高位减一，低位置9

tmp1<= "1001";

tmp2 <= tmp2 - 1;

else --每扫描一次，时间减一，如果低位够减，则只需低位减一,高位不变

tmp1 <= tmp1 - 1;

tmp2 <= tmp2;

end if;

if(tmp3 = "0000" and not(tmp4 = "0000")) then

tmp3 <= "1001";

tmp4 <= tmp4 - 1;

else

tmp3 <= tmp3 - 1;

tmp4 <= tmp4;

end if;

case current_state is --检测当前状态

when s0 => --当现在是南北红，东西绿

north_red <= '1'; south_red <= '1';

north_yellow <= '0'; south_yellow <= '0';

north_green <= '0'; south_green <= '0';

east_yellow <= '0'; west_yellow <= '0';

east_green <= '1'; west_green <= '1';

east_red <= '0'; west_red <= '0';

if(tmp2= "0000" and tmp1 = "0101") then --如果绿灯时间结束

current_state <= s1; --改变状态为南北红，东西黄

end if;

when s1 => --当现在是南北红，东西黄

north_red <= '1'; south_red <= '1';

north_yellow <= '0'; south_yellow <= '0';

north_green <= '0'; south_green <= '0';

east_yellow <= '1'; west_yellow <= '1';

east_green <= '0'; west_green <= '0';

east_red <= '0'; west_red <= '0';

--黄灯显示时间为5

if(tmp2= "0000" and tmp1 = "0000") then --黄灯时间结束

current_state <= s2; --南北绿，东西红

tmp2 <= "0011"; tmp1<= "0000"; --绿灯加黄灯显示时间为30

tmp4 <= "0011"; tmp3<= "0000"; --红灯显示时间为30

end if;

when s2 => --当现在是南北绿，东西红

north_green <= '1'; south_green <= '1';

north_red <= '0'; south_red <= '0';

north_yellow <= '0'; south_yellow <= '0';

east_red <= '1'; west_red <= '1';

east_green <= '0'; west_green <= '0';

east_yellow <= '0'; west_yellow <= '0';

if(tmp4 = "0000" and tmp3 = "0101") then --绿灯时间结束

current_state <= s3; --南北黄，东西红

end if;

when s3 => --当现在是南北黄，东西红

north_yellow <= '1'; south_yellow <= '1';

north_green <= '0'; south_green <= '0';

north_red <= '0'; south_red <= '0';

east_red <= '1'; west_red <= '1';

east_green <= '0'; west_green <= '0';

east_yellow <= '0'; west_yellow <= '0';

--黄灯时间为5

if(tmp4 = "0000" and tmp3 = "0000") then --黄灯时间结束

current_state <= s0; --南北红，东西绿

tmp2 <= "0101"; tmp1 <= "0001"; --红灯时间为50

tmp4 <= "0101"; tmp3 <= "0000"; --绿灯加黄灯时间为50

end if;

end case;

end if;

end if;

end process;

light(0) <= east_red; light(1) <= east_green;

light(2) <= east_yellow;

light(3) <= south_red; light(4) <= south_green;

light(5) <= south_yellow;

light(6) <= west_red; light(7) <= west_green;

light(8) <= west_yellow;

light(9) <= north_red; light(10) <= north_green;

light(11) <= north_yellow;

ew_low<=tmp1;ew_high<=tmp2;

ns_low<=tmp3;ns_high<=tmp4;

end  bhv;