PSoC® Creator™ 101: Lesson 5

Let’s Get an LED to Blink Part 2

July 31, 2026

# Introduction PSoC Creator Pins Part 2	0:00

In the last lesson, we set up two pins. The first was connected to a switch on the board, and the second was connected to an output that was driving an LED. Hopefully, you got the delay to vary using different delay options in your code.  Remember you can always check the course FAQ for help.

# Setting Up an Output Pin	0:27

Controlling an LED from firmware is great, but any micro can do that. Let’s start using more of the PSoC® configurability. We’re going to control an LED from a switch without using any C code at all.

First, let’s re-enable the hardware connection in the LED pin and draw a wire between it and the switch pin.  The wire is green. This tells us that it is a digital connection between the pins. When we build this design, PSoC Creator™ will set up routing inside of the PSoC so that the two pins are directly connected. Try doing that in a fixed-function micro!

We don’t have to write code to set that connection up, and we don’t need the API calls from previous lessons. In fact, there is no software at all.  The PSoC hardware is doing all of the work.

# Adding External Components	1:17

I use a lot of different hardware in my job, and I always struggle to remember whether things like LEDs are active low or active high. I either have to scour through the documentation, or write a little test program to figure it out.

One of the cool features in PSoC Creator is the ability to add documentation to your schematic.  You can show the entire circuit of your designnot just the stuff that’s inside of the PSoC.  I can do this by adding external components to the design that document how the pin is wired on the board. I just enable the external terminal on the pin component. And then I draw in the LED, and the power source or ground, and the current-limiting resistor. Now I’ll never forget that the LED is active low. I do the same thing on the switch, so that I can see that the pin will go low when I press the button.

Notice that the off-chip wires and components are blue.  So you’ll always be able to see that they’re not part of the PSoC.  They are there for your documentation and won’t be built inside of your PSoC.

# Toggling the LED with a Switch	2:26

Let’s take this simple hardware circuit to the next level and show more of PSoC’s flexibility. We’ll make the switch act as an LED toggle.

For this example, grab the toggle flip-flop from the Component Catalog and insert it between the two pins. Attach logic 1 to the flip-flop input, and the LED will now toggle off and on when you press the button.  How cool is that?

This is a pretty simple example of how we can modify the signal. When you’re more familiar with the PSoC, you’ll soon be routing signals through multiplexers and different kinds of gates. You’ll be able to create sophisticated state machines with our look-up table component and, in some of our next lessons, you’ll be controlling functions with a pulse-width modulator and with timers.

# Next Steps: First Challenge	3:16

Now it’s time for your challenge. If you did the software project from the previous lesson, and you should have, try working that to replace the firmware with the flip-flop. Once you have that working, try driving a second LED.  The green LED is attached to P0[2] on the Pioneer board, but reverse the order of the toggling so the light switches between the blue and the green.

# Using the PWM Component	3:46

For the next example, we will control the LED using one of the PSoC’s built-in PWMs and a clock.  For this project, we will also enable the blinking LED using Pioneer’s built-in hardware switch.

First place the PWM component.  Double-click on the PWM component to start the device configuration dialog: click on the button that enables the HW enable; set the period to 100 and the compare to 50; and name the component PWM.

In this example, the 100 will divide the input clock by 100 yielding a 1 Hz output and the 50 in the compare value will set the duty cycle to be 50/50. Next, place a Clock component.  Double-click on the clock, and set the input frequency to 100 Hz.

Last, wire the schematic together by connecting the switch to the PWM input, the LED to the PWM output, and the clock component to the PWM clock.  The last thing that you will need to do is add firmware to start the clock and to start the PWM.  You will do that with the built-in component feature Start.  Lastly, build and program the PSoC. You will now have a functioning, switch-controllable, blinking LED.

# Next Steps: Second Challenge	3:46

Now, there are some things that you can do to make this your own project.  And as a challenge, change the design to blink when the button is pressed, instead of when it’s not pressed.

Also, try changing the duty cycle of the LED by changing the PWM parameters.  This can be accomplished by changing the compare value and the period value of the PWM.

In the previous examples, you used the hardware of the PSoC to control the switches and the LEDs.  In the coming example, you’ll be using Cypress’s proprietary CapSense to control projects.

Cypress Semiconductor 198 Champion Ct. San Jose, CA 95134-1709 USA Tel: 408.943.2600 Fax: 408.943.4730 Application Support Hotline: 425.787.4814 www.cypress.com

© Cypress Semiconductor Corporation, 2014. The information contained herein is subject to change without notice. Cypress Semiconductor Corporation assumes no responsibility for the use of any circuitry other than circuitry embodied in a Cypress product. Nor does it convey or imply any license under patent or other rights. Cypress products are not warranted nor intended to be used for medical, life support, life saving, critical control or safety applications, unless pursuant to an express written agreement with Cypress. Furthermore, Cypress does not authorize its products for use as critical components in life-support systems where a malfunction or failure may reasonably be expected to result in significant injury to the user. The inclusion of Cypress products in life-support systems application implies that the manufacturer assumes all risk of such use and in doing so indemnifies Cypress against all charges. PSoC Designer™, Programmable System-on-Chip™, and PSoC Express™ are trademarks and PSoC® is a registered trademark of Cypress Semiconductor Corp. All other trademarks or registered trademarks referenced herein are property of the respective corporations. This Source Code (software and/or firmware) is owned by Cypress Semiconductor Corporation (Cypress) and is protected by and subject to worldwide patent protection (United States and foreign), United States copyright laws and international treaty provisions. Cypress hereby grants to licensee a personal, non-exclusive, non-transferable license to copy, use, modify, create derivative works of, and compile the Cypress Source Code and derivative works for the sole purpose of creating custom software and or firmware in support of licensee product to be used only in conjunction with a Cypress integrated circuit as specified in the applicable agreement. Any reproduction, modification, translation, compilation, or representation of this Source Code except as specified above is prohibited without the express written permission of Cypress. Disclaimer: CYPRESS MAKES NO WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, WITH REGARD TO THIS MATERIAL, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. Cypress reserves the right to make changes without further notice to the materials described herein. Cypress does not assume any liability arising out of the application or use of any product or circuit described herein. Cypress does not authorize its products for use as critical components in life-support systems where a malfunction or failure may reasonably be expected to result in significant injury to the user. The inclusion of Cypress’ product in a life-support systems application implies that the manufacturer assumes all risk of such use and in doing so indemnifies Cypress against all charges. Use may be limited by and subject to the applicable Cypress software license agreement.